User Guide for NeuronID

Jikan Peng (pengjikan@westlake.edu.cn) | Westlake University, China

#### 1. Overview

NeuronID is an open-source, automated toolkit for processing two-photon calcium imaging data, featuring motion correction, noise reduction, neuronal component segmentation, and signal extraction. This guide provides instructions for installing and running the software.

#### 2. System Requirements & Installation

- Required Software: MATLAB R2023b or later (MathWorks, USA).

- Installation: No formal installation is required. Simply add the NeuronID directory and its subfolders to your MATLAB path.

- Launching the Application:

  - GUI Mode (Recommended): Type CSE in the MATLAB command window, or directly run CSE.mlapp.

  - Code Access: To inspect or modify the source code, open CSE.mlapp in MATLAB’s App Designer.

#### 3. Data Preparation

We recommend organizing your data as follows for optimal compatibility:

- Create a main folder for each subject (e.g., Mouse#1).

- Inside this folder, create a subfolder named Raw to store all original two-photon imaging files (e.g., .tif stacks).

- Ensure you have the following metadata for your imaging data:

  - Pixel Dimensions (width x height in pixels)

  - Field of View (physical size in micrometers, e.g., 512 µm x 512 µm)

  - Imaging Frequency (frame rate in Hz)

#### 4. Operation Modes

NeuronID offers two primary analysis modes:

- A. Single-Subject Analysis

  - Click Open Mouse File and select the subject's main folder (e.g., Mouse#1).

  - Input the required parameters:

    - Scan Scale Ratio: (Physical Size per Pixel) = Field of View (µm) / Pixel Dimension.

    - One F Duration (ms): Time to acquire one frame = 1000 / Imaging Frequency (Hz).

  - Click Calculate to compute the calcium indicator's decay constant.

  - Click One-Click Run (Mouse) for a fully automated analysis pipeline.

- B. Batch Analysis (Multiple Subjects)

  - Ensure all subject folders are within a single parent directory.

  - Prepare a mouse.xlsx file listing the information for all subjects (a template is provided in the Mice folder).

  - In the CSE.mlapp source code, modify the Path variable for the One-Click Run (Mice) button to point to your parent directory.

  - Click One-Click Run (Mice) to process all datasets automatically.

#### 5. Step-by-Step Module Description

For users who wish to execute and validate each step manually, the workflow is broken down into the following modules:

- 5.1. Motion Correction

  - Function: Corrects for rigid and non-rigid motion artifacts.

  - Action: Click the Motion Correction button.

  - Algorithm: Based on the NoRMCorre algorithm (https://github.com/flatironinstitute/NoRMCorre) with custom modifications.

  - Output: Corrected images are saved in the Registration folder.

  - Note: This step requires significant internal storage. Please ensure adequate disk space.

- 5.2. Noise Reduction

  - Function: Reduces independent noise using a deep learning approach.

  - Algorithm: Based on the DeepInterpolation algorithm (https://github.com/AllenInstitute/deepinterpolation；https://github.com/MATLAB-Community-Toolboxes-at-INCF/DeepInterpolation-MATLAB).

  - Workflow:

    - Normalize Parameters & Extract Key Frames: Prepares data for training.

    - Train Personal Model: Performs transfer learning from a pre-trained model (default: 30 Hz). Adjust the Number of Epoch as needed.

    - Evaluate Personal Model: Visualizes the training loss curve.

    - Denoise Images: Applies the model to reduce noise.

  - Quality Control: Use SNR after Denoise and Evaluate Denoise to assess performance.

  - Pre-trained Models: Multiple models for different imaging frequencies are available in the AIModel folder. NeuronID will auto-select a compatible model if available.

  - Output: Denoised images are saved in the Denoise folder.

- 5.3. Segmentation of Neuronal Components

  - Function: Identifies and segments individual neuronal somata.

  - Workflow:

    - Initialization: Prepares the data for segmentation.

    - Periodic Masks: Generates preliminary masks from temporal blocks (Default: Window Size=100, Step Size=20).

    - Potential Location: Identifies and excludes neuropil-associated pixels.

    - t-SNE Projection & Central Pixel: Clusters pixels by similarity and identifies key pixels (Default: Number of Center Pixels=9,000).

    - Temporal Mask: Integrates information across time to generate a temporal mask.

    - Mask Overlap & Combine Masks: Resolves overlapping regions.

    - Spatial Mask: Produces the final somatic mask.

  - Output: All masks and intermediate results are saved in the Segment folder.

  - Note: This step is computationally intensive and requires sufficient internal storage.

- 5.4. Extraction of Neuronal Signal

  - Function: Extracts and processes fluorescence signals from segmented neurons.

  - Workflow:

    - Neuronal Fluorescence & Neuropil Fluorescence: Calculates raw signals.

    - Correct Fluorescence: Removes neuropil contamination and corrects for photobleaching.

    - Calcium Signals: Calculates ΔF/F to generate the calcium signal.

    - Event Signals: Performs deconvolution to infer spiking activity, using the provided Decay Index.

  - Output: Extracted signals are saved in the Signal folder.

#### 6. Utilities

- Timeline Export: Use View Frames and Export Timeline to export temporal metadata for each frame.

- Mask Evaluation: The Evaluate Mask button allows for quantitative comparison between different segmentation masks.

#### 7. License, Copyright, and Citation

- Copyright: The overall architecture, integration code, and original modules of NeuronID are Copyright (c) Jikan Peng, Westlake University.

- Third-Party Code: This toolkit incorporates and modifies open-source components, including but not limited to:

  - NoRMCorre: Copyright (c) their respective authors. Source: https://github.com/flatironinstitute/NoRMCorre.

  - DeepInterpolation: Copyright (c) their respective authors. Source: (https://github.com/AllenInstitute/deepinterpolation；https://github.com/MATLAB-Community-Toolboxes-at-INCF/DeepInterpolation-MATLAB).

- Use: Use of NeuronID is permitted for non-commercial academic and research purposes only. For any commercial use, please contact the corresponding author.

- Citation: If you use NeuronID in your research, please cite our publication.